Novel viscoelastic model of IEAP actuators.

Veix, Punn, Alvo

Abstract tuleb alles siis kui conclusion on valmis.

The innovatory viscoelastic model of bending ionic electroactive polymer actuators allows modelling

1. Introduction

Already since the very first reports concerning the bending ionic electroactive polymer actuators, the researchers have described the „back-relaxation“ effect – an actuator excited with DC voltage, instead of holding its bent state, relaxes slowly back towards its initial shape. This behavior is commonly treated as a shortcoming of IEAP actuators, decreasing their ability to deliver a constant peak force to target, limiting their frequency range, and hindering their exact control. There have been several attempts to slower the rate of back-relaxation by choosing an opportune combination of separator, electrodes or mobile ions. Nevertheless, there is no report claiming that in the case of some IEAP material this holdback is completely eliminated.

Supposedly the phenomenon of back-relaxation is studied almost only in the case of „old-type“ ionic polymer-metal composites (IPMC). It is a wet ion exchange polymer membrane covered with thin metal electrodes on both sides. [X7] For typical IPMC with platinum electrodes, when a voltage is applied across the thickness direction, the electromechanical transduction occurs due to the motion of the cations along with the water molecules. As a result, one face of IPMC withers and shrinks and the opposite face protracts due to swelling with water and ions. This generates warping of the whole laminate. The resulting fast bending motion toward the cathode is followed by a slow relaxation towards the anode, while IPMC as the whole still remains bent. If now the input of this actuator is shorted, the actuator performs a sharp movement towards its initial state, but will usually outdistance. The steps of this process are depicted in Fig. 1. The speed of back-relaxation is reported being dependent on the type of polymer membrane, solvent, cations, etc. [X6]. It is commonly believed that the slow relaxation happens due to the water diffusion out of the strained polymer matrix. [X7] Most of the proposed electromechanical models describing the behavior of IPMC, just neglect the phenomenon of back-relaxation. [XXX] One of the few exceptions is presented by Bao et al. [X3], proposing the model of IPMC without and with the back-relaxation by adding a separate relaxation time constant. [X8] kirjeldab ka midagi.

Siia teeme uued fotod kuidas üks aktuaator tagasi vajub. Kell juurde!

Fig. 1. Actuation of IPMC. Forward bending is followed by back-relaxation.

In recent years the range of IEAP materials has been significantly extended by means of materials used for electrodes, separator membrane, as well as the electrolyte. At first glance, all ionic EAPs seem similar in construction – two conducting electrodes separated by a polymer membrane, containing freely moving ions – but their actuation mechanisms can be significantly different. There haven’t appeared any definitive models or at least reports yet, describing the phenomenon of back-relaxation of IEAP actuators with electrodes based on carbon, or conductive polymers.

Experimenting with the IEAP actuators with carbon electrodes, we have noticed that similarly to the water-containing IPMC, virtually all of them exhibit back-relaxation, at least to some extent. It appears that by means of their transient spatial actuation and transient moment of force, the long-term behavior of all IEAP actuators is just similar. The back-relaxation is present regardless of the absence of the fluent liquid, and regardless of the membrane or electrode material.

The bending-relaxing behavior of the IEAP actuators resembles viscoelasticity, but seems working partly conforming, partly contra to the electrical excitation. For this behavior the constitutive models of viscoelasticity, consisting of linear combinations of springs and dampers, are inappropriate. There exist numerous models describing the material’s response in different loading conditions, e.g. Maxwell model, Kelvin-Voigt model, or combinations of both of them. [XX] All the models describe only the response of the material to the external load. Alas, neither of them involves any element causing the actual behavior of the IEAP actuators – lapsing against the load. The cause of this failure is the concept of the so-called canonic models of viscoelasticity [XX], where the load is external, and is applied to the whole system.

The objective of the current paper is describing the time-dependent correspondence between the transient input voltage and the shape of the IEAP actuators, taking into account also the back-relaxation. We ground to the scalable distributed model of IPMC, presented in [DM], and extend it to different IEAP materials. This approach describes the commonly uneven propagation of voltage along the IEAP material, and associates the voltage and charge carried over with the uneven flexure of the actuator. The experiments described in [DM] assume that the charge-flexure relation is simply linear, and leave off just when the phenomenon of back-relaxation becomes noticeable. The current paper associates our novel model of viscoelasticity with [DM]. The resulting electromechanical model of IEAP allows describing the shape or blocking force of the actuators over a long time. Its transient input signal is not limited in any way, due to the accordance to the Boltzmann superposition principle – each loading step makes independent contribution to the total loading history [ISBN: 1569903484].

This paper is organized as follows: Firstly, in Section 2 we show that the innovatory approach to an assembly of springs and dampers will result with PDE-s describing the truthful bending of the actuators forwards and backwards under a constant exciter. In Section 3 we demonstrate the capabilities of the proposed model of viscoelasticity with the response of a lumped model of a fictional IPMC. The thorough characterization of the mechanics of IEAP actuators given in Section 4 is inevitable for understanding the distributed electromechanical model introduced in Section 5. The verification of the model with the examples of three completely different IEAP materials.is presented in Section 6. The discussion in Section 7 disputes about the further improvements of this representation.

2. The model of viscoelasticity with internal strain.

As mentioned hereinabove, in the canonic models of viscoelasticity the load is external, and is applied to the whole combination of springs and dampers. This configuration does not involve even any possibility to suppress the exciter. The situation changes completely, when the exciting factor is applied between the spring and damper, resembling something expanding-contracting between the elastic threads of the polymer molecules network. The viscoelastic scheme is depicted in Fig. 2. It consists of a damper in series with our new element - internal strain -, both in parallel with a lone spring. The internal strain is a vector of variation of the length of a non-elastic element. Hereinafter we associate it with the transient electrical charge q of the IEAP material, so for clarity we label the strain element with Q and portray with Q between arrows. When internal strain εq of the element Q is applied, the spring reacts momently, while the damper follows with time, tending towards the unstressed state of the spring. In brief, the input of our viscoelastic element is the strain εq, while the output is the strain of the spring εs. Currently, we only focus on the time dependency of the model designating the characterization of frequency response as future work.

Fig. 2. The model of viscoelasticity with internal strain.

The behavior of this viscoelastic model in different loading conditions is depicted in Fig. 3. We insist that there are no other external stresses or strains affecting the system besides εq, indicating that σTotal=0. Initially the spring and damper are in their unstressed state: σd=σs=0 (A). The positive strain of Q draws the spring out (B) while the damper releases in time. This process continues regardless of the applied strain Q until the stress of spring is vanished (C). The removal of the strain Q results with the compression of the spring (D), followed by gradual dragging of the damper, until the stress of the spring is again zero (E). The negative strain of Q is depicted as its shortening (F) and compression of the spring. Similarly, the spring draws the damper longer until reaching the unstressed state (G). As seen in this figure, this system strives to gain its unstressed state with any stress Q.

Fig. 3.

The definition of the model is summarized with four relations in (1).

|  | σTotal=σDt+σS(t)=0εTotal=εS(t)=εD(t)+εQ(t)σS(t)=EεS(t)σD(t)=ηdεD(t)dt |  |
| --- | --- | --- |

where E and η are modulus of elasticity of the spring and viscosity of the damper respectively. We see two equivalent solutions to this model: with respect to the input strain εQ and with respect to the derivative of the input strain εQdt, here we present both of them. The described relations in (1) can be arranged into differential equation for strain of damper.

|  | ηdεD(t)dt+EεD(t)+EεQ(t)=0. |  |
| --- | --- | --- |

The solution for εS is found by bearing in mind εS=εD+εQ and solving the first order linear differential equation with homogenous initial condition εD0=0.

|  | εSt=εQt-Eη e-Eηt0tεQτeEητdτ. |  |
| --- | --- | --- |

A different form of the equation is derived by combining (1) for strain of the spring directly

|  | EεS(t)+ηd[εSt-εQt]dt=0 |  |
| --- | --- | --- |

Solving (4) with εS0=0 gives

|  | εS(t)=e-Eηt0tddξεQξeEητdτ |  |
| --- | --- | --- |

This form might be useful if the derivative of εQ is known. By introducing the rate of relaxation λ=Eη and expressing the input strain in terms of stress εQ=σQ(t)E both given forms are simplified to (7) and (8)

|  | εSt=1EσQt-λe-λt0tσQ(τ) eλτdτ. |  |
| --- | --- | --- |
|  | εSt=1Ee-λt0tddξσQξeλτdτ |  |

3. Lumped model of IEAP with relaxation:

In order to demonstrate the capabilities of the proposed model of viscoelasticity and its correspondence to the superposition principle, we use a simple lumped model of IPMC presented e.g. in [Bao]. This concept couples the input voltage with transferred charge and the tip displacement of an IPMC actuator. It gives reasonable results in case of small displacements only and serves well to explain the complicated approach presented in the next sections of this paper. Here we just demonstrate the transient response of the model to a rectangular input signal without attempting to validate it with any real actuator.

The lumped model of IPMC is presented in Fig. 4A while the electrical signals as well as strains are depicted in Fig. 4B. The correspondence between the input voltage Uin(t) and charge q(t) is

ValemZZ

and the correspondence between the strain (or tip displacement) and charge is linear:

ε(t)=k*q(t) .

The transient charge q(t) (green in Fig 4B) corresponds to the rectangular pulse of input voltage Uin(t) (red in Fig. 4A). Two, according to (7) numerically modeled transient strain behaviors with different values of λ and convenient values of the coefficient k to fit the graphs into the picture, are presented as blue and yellow graphs.

Fig. 4. Lumped model of IPMC.

Supposing that initially the system has been shorted for long time, the initial charge of C is zero: q(0)=0. As seen in Fig. 4., when voltage Uin is turned on, the strain ε(t) initially follows the charge q(t), in certain moment turns relatively suddenly back, then relaxes slowly with the speed determined by λ. When the voltage is switched off, the actuation towards the initial state again keeps the curve of charge followed by back-relaxation, but according to the superposition principle starts from the momentary values of strain and charge. Similar back-relaxing behavior of IPMC is described in numerous papers e.g. in [Bao, XX1,XX2,XX3].

4. Distributed bending moment of the actuator

4.1. Characterization of the shape of the bending actuator

Through the time, mainly two different representations of the shape of IEAPs have been exploited – the tip displacement and the curvature. Last of which is more commonly associated with actuators exhibiting large deflections. Often the shape of the actuator is represented by a single radius of curvature which is justified for only very specific type of IEAPs. As shown in this paper, due to different physical phenomenon, the actual curvature can vary significantly along the axis of IEAP. This in turn indicates that a form of distributed representation of curvature is essential for accurate modeling of the actuators.

An already published technique, which satisfies the requirements described above, is to split the curve into segments or vectors (FIGX)[X13]. Although, this representation preserves the exact information about the endpoints of each segment, the curvature in between the points is reduced to a constant. Nevertheless, the desired accuracy can be reached by increasing the count of segments or decreasing the length of a segment.

The vector representation of a curve can be obtained by detecting the extremums of pixel intensities as indicated in figure 5. The curvature is found directly from its definition κ=dφds considering ds=rarc. Applying the procedure for each frame of the video results a 3-dimensional representation of curvature along the actuator in time (Fig6). Since the segments are not depending on each other we are easily able to interpolate along the coordinate to obtain a continuous curve of the actuator. In this work, the vector representations of IEAPs were attained by NI LabView image processing tools.

| Fig. 5 Transformation to vector representation. | Fig. 6. Transient behavior of the actuator. Kui saaks siia tagasivajumise ka? |
| --- | --- |

4.2. Characterization of the bending moment of the bending actuator

The conventional IEAP can be observed as cantilever beam with fixed end attached to contacts (FIGX). In this configuration bending moments can be separated as follows: An electrically induced bending moment EIBM, a moment Mw caused by the weight of the actuatorMw, and an internal bending moment Mr countering the sum of EIBM and Mw. In this work we have neglected the weight of the actuator and Mw by holding the sample such that gravity vector and motion vectors of IEAP are always perpendicular i.e. holding the actuator edgewise. Because of the transient approach, we investigate each time step as an independent static problem and the Euler-Bernoulli beam theory can be exploited to relate the curvatures with the corresponding bending moments.

dφds=M(s)EI

Where E is the modulus of elasticity and I is the second moment of area. Since the cross-section of samples in current work is a rectangle, the second moment of area is expressed as I=bh312 where b and h are width and height of the cross-section of the sample respectively. Though the value of I could be directly calculated for every particular IEAP, it could be difficult to attain acceptable accuracy due to the highly amplified errors originated from the determination process of the height member.

5. Viscoelastic electromechanical model of IEAP actuators

5.1. Previous work

The concept of the distributed electromechanical model of IEAP actuators is introduced in [DM]. This approach declares that the bending of an IPMC actuator at any time and any point is caused by the charge carried over between the electrodes at that point by that time. The amount of charge is determined by the inhomogeneous transitory voltage that in turn is uneven due the electrical resistance of the electrodes. The electrical constituent of that model depicted in Fig. 10. resembles a sophisticated lossy RC transmission line. The conductivity of the electrodes of the IEAP material is represented by a series of resistances of the opposite electrodes Ra and Rb, while the capacitance C, and the loss parameters G form the impedance of the material.

Fig. 10. IEAP resembling a lossy RC transmission line. Siia Cauer!

Although that paper emphasizes the flexure of the IPMC actuator is non-uniform in spatial and time domainand and that it is scalable by means of the the length of the actuator, it is contented with the simple proportional relation between the flexure and the charge, and discontinues just when the phenomenon of back-relaxation becomes noticeable. The objective of the current paper is to proceed from that stage of development.

One of the advantages of the approach of RC transmission line is the presence of the analytical solutions of its response to some specific input signals, particularly to the Heaviside step function. This enables easy comparison of the model with the real response of the actuator to the input voltage signal representing a flat step up. Moreover, it is possible to attach additional contacts to the surface of the actuator, and in this way validate the electrical parameters of the transmission line [XX, XX]. The propagation of voltage along this RC line is described by the following PDE:

.		(V1)

It is a pseudo-parabolic PDE, similar to those describing e.g. diffusion in porous media or heating a rod with radiation and loss present. Fortunately, it is possible to solve it analytically for u(x,t) utilizing the method of separation of the variables [DM, RCGWLine]. Moreover, by applying the initial and boundary conditions – initially the line is discharged and its other end is open – the analytical expression describing the time-evolution of the voltage distribution on the line after applying a voltage step to its input is

(V2)

where L is the length of the sample,

, and .

As seen, the complicated time-evolution of the voltage distribution on the line is defined solely by the measurable parameters: by impedance of the material, by the length of the sample, and by time. Although the PDE describing the transient transferred charge is analogous to (V1), it is impossible to apply the boundary conditions and obtain the solution, similar to (V2). Instead, if initially the line is discharged, the charge is expressed via voltage as

.				(V3)

As an illustrative example, the transient behavior of voltage and charge of the actuator depicted in Fig. 1., are given in Fig. 11.

Fig. 11. A – voltage; B – charge q

5.2. Distributed viscoelastic electromechanical model of IEAP actuators.

The illustratory diagram of our viscoelastic electromechanical model of IEAP actuators is given in Fig. 12. It consists of infinite series of infinitesimally short lumped units, similar to those described in Section 3, while the conductive opposite electrodes of the IEAP material assure the electrical conductivity between the lumped units. Its electrical equivalent circuit is a lossy RC transmission line, described in the previous section, while the mechanical constituent stands of series of infinitesimally short hinged links. The buffers between the links are the models of viscoelasticity with internal strain, presented in Section 2. Even though the bending motion of IEAPs is caused by different physical processes, the relationship between the transferred charge and the input of the viscoelastic scheme is considered similarly linear by α for all of them.

Fig. 12. The coupling between equivalent circuit and mechanical constituent.

Considering the modulus of elasticity E and the second moment of area I as constants allows us to introduce a constant μ=αEI and rewrite (4) for curvature.

|  | 1ρ=μqt-λe-λt0tq(τ) eλτdτ. |  |
| --- | --- | --- |

The curvature can be also presented in Cartesian coordinates as:

curves=0scos⁡(φs)ds+x0  0ssinφsds+y0

where x0 and y0 are the initial position of the curve and the angle

φs=0sρsds+φ0

where φ0is initial angle at s=0.

The parameters of the distributed electromechanical model were determined using differential evolution algorithm. [X12] The cost function was defined by simply minimizing the differences between measured and calculated voltages.

Hoolimata sellest missugune füüsikaline protsess liigutuse tekitab, kas elektrostaatiline tõukumine või mehhaaniline vahelepressimine, igal juhul me seostame liigutuse transferred charge-ga ning oletame et see seos on lineaarne.

Surface resistance of the sample was measured experimentally … (directly/ four point method kõlaks siin hästi?) All other parameters were obtained by DE algorithm. Since the model is not yet fully optimized its evaluation in the meaning of reasonable precision is still computationally expensive. Therefore, the optimization of each sample was carried out in two stages. First, the parameters C, G, W and μ were determined using fixed value of λ=0. In other words, we assume that at the beginning of the actuation there is no relaxation present. In second stage, all other parameters were fixed and the optimization was performed respect to λ only. Described two stage optimization gives an advantage to determine λ with coarse set of experimental points while the C, G, W and μ are determined with fine set of points over shorter time period.

|  | CPC with gold electrodes | CPC | musclesheet |
| --- | --- | --- | --- |
| Dimensions |  |  |  |
| R | 2 | 1600 | 9 |
| C | 0.468 | 0.00585 | 0.0089 |
| G | 0.035 | 0.0002 | 0.0063 |
| W | 1.889 | 0.0193 | 0.6044 |
| μ | 0.10365 | 6.52039 | 8.7504 |
| λ | 0 | 0 | 0 |

6. Verification of the model

Kõik eespooltoodud graafikud on Pt-IPMC jaoks. Siin näitame et kõik tulemused kehtivad ka 2 täiesti erineva IEAP materjali korral. Materjalide kirjeldused, võta Janno artiklist ja viita et autori suulisel loal. Miks me võime oletada et elektriskeem ikka kehtib? Mis on CPC korral RCGW. Mingis Karli artiklis on et Karl ei leidnud elektroodi seest mahtuvuslikku komponenti.

The parameter λ was derived from eq. 5 and was evaluated in region 120s<t<300s.

| Parameter; Value. R; 1.6kΩcm. C; 0.011Fcm. W; 5.11kΩ∙cm. G; 0.0981kΩ∙cm. Table 1. Measured parameters for CPC actuator. | Fig. 4 Measured and simulated voltage along the CPC at 2.0 V step input signal. |
| --- | --- |

As seen in figure 1, good correlation between experimental data and simulated data with optimized parameters was obtained.

The simulated points are clearly following the actual shape of the actuator. The shape of the actuator

| Fig. 5 Measured and modelled shapes of the actuator at 2.5V. | Fig. 6. Measured and modelled shapes of the actuator at 2.5V |
| --- | --- |

7. Discussion

We have introduced a novel viscoelastic model for representation of bending motion of different types of IEAPs. The two equivalent forms of the model were derived out of which the first one was demonstrated as part of a distributed electromechanical model. However, there is no reason to underestimate the alternative form of the model which allows the substitution of the charge with corresponding current for instance.

Even though the model showed a good performance with the three actuators, there are still several currently ignored issues to examine:

- In this model we assume constant resistance of the electrodes, still it is clear that the extension and compression of the electrodes may noticeably affect the resistance;

- The deformation is modeled in accordance to Hooke’s law and is not covering the creep and the permanent deformation occurring beyond the elastic limit.

- Actually there are 3 elastic modulus and using one effective young modulus is appropriate only in the case of symmetric IEAP.

- Internal friction?

Mida kõike meie mudeliga teha saaks.

Sobib ka muuks. Mõtle näiteid. Näiteks on suur problem supercäppide liikumine. Indreku artiklis on öeldud et aktuaator, sensor ja supercäpp on ehituselt samad. Seega aitaks meie mudel kindlasti supercäppe modelleerida.

Acknowledgments

Refs

X1: doi: 10.1117/12.654740

X2: doi:10.1088/0964-1726/17/3/035011

X3: doi: 10.1117/12.475167

X4: doi: 10.1002/marc.201100535

X5: doi: 10.1002/pen.21955

X6: doi: 10.1063/1.2194127

X7: doi:10.1088/0964-1726/20/8/083001

X8: doi: 10.1117/12.475201

X9: PunnDistrModel

X10: doi: 10.1021/ma800956v

X11: Spie 2002 - 4695-33

X12: DOI: 10.1023/A:1008202821328

X13: doi:10.1088/0964-1726/18/11/115023